|  | Simulation of shaft2 Date: 23 فبراير, 2016 Designer: Solidworks Study name: Static 2 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: shaft2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Extrude3; Solid Body; Mass:1.39029 kg Volume:0.000176927 m^3 Density:7858 kg/m^3 Weight:13.6248 N; E:\SolidWork\Project\shaft2.SLDPRT Feb 23 03:37:33 2016.

# Study Properties Study name; Static 2. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (E:\SolidWork\Project).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 1023 Carbon Steel Sheet (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.82685e+008 N/m^2. Tensile strength:; 4.25e+008 N/m^2. Elastic modulus:; 2.05e+011 N/m^2. Poisson's ratio:; 0.29. Mass density:; 7858 kg/m^3. Shear modulus:; 8e+010 N/m^2. Thermal expansion coefficient:; 1.2e-005 /Kelvin.; SolidBody 1(Cut-Extrude3)(shaft2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.0197661; -1.23212; 0.766886; 1.45142. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 3 face(s). Type:; Apply normal force. Value:; 1 N.. Torque-1; Entities:; 1 face(s). Reference:; Axis1. Type:; Apply torque. Value:; 1 N.m..

# Connector Definitions Pin/Bolt/Bearing Connector Model Reference; Connector Details; Strength Details. Bearing Support-1; Entities:; 1 face(s). Type:; Bearing.; No Data. Connector Forces Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0.074818; -3.1462e-018; 0; 0.074818. Shear Force (N); -8.3267e-017; 1.9742; -0.87241; 2.1584. Bending moment (N.m); -1.#IND; -1.#IND; -1.#IND; -1.#IND.. Bearing Support-2; Entities:; 1 face(s). Type:; Bearing.; No Data. Connector Forces Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0.055054; 2.3151e-018; -0; -0.055054. Shear Force (N); -1.3878e-017; 0.26379; 0.10841; 0.28519. Bending moment (N.m); 0; 0; 0; 0..

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 5.61553 mm. Tolerance; 0.280777 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 14731. Total Elements; 9396. Maximum Aspect Ratio; 7.846. % of elements with Aspect Ratio < 3; 96.2. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; MONGOOSE-PC.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0197661; -1.23212; 0.766886; 1.45142. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 759.699 N/m^2 Node: 9656; 1.28251e+006 N/m^2 Node: 14250. shaft2-Static 2-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 mm Node: 50; 0.000366607 mm Node: 659. shaft2-Static 2-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 5.58685e-009 Element: 7561; 3.59695e-006 Element: 2942. shaft2-Static 2-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. shaft2-Static 2-Displacement-Displacement1{1}.

# Conclusion